Mt. Everest

Group Project report

ASSIGNMENT COVER SHEET

| Course: BEng Computing (CN) |  |  |  | Year: 2 |  | CSY2027 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group Project |  | Title: Design and Development of departmental computer network for Engineering and Technology Department |  |  |  |  |  |  |
| Date due out: 16th Oct 2020 | Date due in: 2nd May 2021 |  | Extension date: |  |  |  | Extension agreed by: |  |
| Student Names (List each member of the group):Mt. Everest Kaushal Ghimire, Ashok Ghimire and Basant Karki |  |  |  |  |  |  |  | Tutor: Suresh Gautam |
| Student comment, specific request for feedback etc. |  |  |  |  | Marker’s General View of the work |  |  |  |

ASSESSMENT FEEDBACK:

| RATING SCALE | A (A-, A, A+) (Excellent) |  |  |  | B(B-, B, B+) (Good) | C (C-, C, C+) (Satisfactory) |  | D (D-, D, D+) (Needs some more work) | F (F-, F, F+) (Needs much more work) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Requirement Specifications Documentation |  |  |  |  |  |  |  |  |  |
| System Design Documentation |  |  |  |  |  |  |  |  |  |
| Prototype Functionality |  |  |  |  |  |  |  |  |  |
| Testing/Evaluation Strategy |  |  |  |  |  |  |  |  |  |
| Group Cohesion, Teamwork and Project Management |  |  |  |  |  |  |  |  |  |
| System Presentation and Demonstration |  |  |  |  |  |  |  |  |  |
| Specific aspects of your assignment that the marker likes; |  |  | Specific aspects of your assignment that need more work: |  |  |  |  |  |  |
| Tutor’s Signature: |  | Date: |  |  |  |  | Grade: |  |  |

Table of Content

- Acknowledgement

- Abstract

- Introduction

- Project Background/pre-installed system analysis

- Aims and Objectives

- Literature review

- Development Methodology

- Requirements Engineering

- Elicitation Activities

- Interview Plans and questions

- Interview finding

- Other Problem Domain

- Evaluation of Existing system

- Comparable Software System Review

- Requirements Specification

- Problem domain description

- Functional Requirements

- Performance Requirements

- Security Requirements

- Testing Requirements

- Hardware and software requirements

- Network and web Design

- Network Topology design

- Website application designed mockup

- Implementation

- Network Implementation

- VLAN

- VTP

- Trunking Protocol implementation

- Vlan Traffic load balancing (VLAN HSRP)

- DHCP Helper IP VLAN interface

- EtherChannel Port channel configuration

- Dynamic routing configuration (using EIGRP):

- NATTING

- VPN implementation

- Windows server configuration

- Active directory domain service (AD DS)

- Domain name system (DNS)

- DHCP (Dynamic host configuration protocols)

- Web Server

- References

- Appendix

Acknowledgement

- Introduction

The engineering department of Avenue campus, which is located in Northampton, is requiring an additional network infrastructure for better improvement and learning experience of students and develops in their fields. Whereas, IT services located in park campus are providing services and managing the whole networking system. The computing department of the school of science and technology has worked hard to develop and manage their servers and facilities, so that they can maintain their department and school differently. The present system is used for supporting student work and staff as well.

Some examples of current running servers are web server, file server and authentication server in the computing department.  Similarly, in this network staff can easily manage their own sections and use the engineering department website for uploading their modules materials and their weekly assignments. The overall aim and objective of the project is to study the existing system and find out the drawback and bring the possible change and improvement for the betterment of the department of engineering and technology. Whereas, The new system will be more user-friendly, and reliable for management, as well as students of the computing department, which will be designed and implemented for the department of engineering and technology.

- Project Background/Literature Review

As the investment for this project of designing and configuring the network prototype is large, the university is mainly focused on security, redundancy, stability, and Scalability of the network for enhancing the experience of student’s learning In Networking section. And for the web section. The network design will be fully designed under the protocols and rules of cisco 3-layer hierarchical model. Similarly, in this network, staff can easily manage their own sections and use the engineering department website for uploading their modules materials and their weekly assignments.

Just like other departments, like departments of arts, science and technology; the department of engineering and technology will manage its own servers, like: file and web (www) server by its own to make it easier for the students and the staff of the computing department. Currently, the present website of computing departments (i.e. www.engtech.northamptan.ac.uk ) is hosted on the www server and WAMP server is installed on the same server for supporting the students for Apache, MYSQL, and PHP. WAMP is a kind of server tool which is used for web-development, serving live websites and internal testing for windows systems. The complete word of WAMP ‘windows,Apache,MySQL and PHP’. The web developer uses Apache for testing their website.

- Aims and Objectives

The main purpose of the project is to analyze, finding low-sides and re-design, implementing and testing the overall network, by applying the implementation of a new designed prototype network on the basis of a 3-tier/layer cisco hierarchical model for the engineering and technology department of university of Northampton. Similarly, in this network, all the staff will easily manage their own sections and will use the engineering department website for uploading their modules materials and their weekly assignments.

The main objectives of the project are listed below:

- To design a new prototype for the network of engineering and technology department.

- To install the its own servers, like: file and web (www) server as well as Authentication server

- To make internet access (wire and wireless) available to the students and staffs.

- To allow module tutor/leader to make change, upload, delete, and edit the content on both web server and fileserver.

- To use RADIUS for the wireless authentication and make strongly secured network.

- To make privileges group  for different  users like, admin, staff and students

- To make the server accessible from the remote area.

- To make overall network more secure and bug free.

- To monitor the network performance using different tools.

- Development Methodology

Our team has been assigned the works to complete this project. The focal point that acts as the main goal of the project is to design and implement a prototype network model for the Engineering and Technology department based on a three layer Cisco hierarchical model. Similarly, it will also be focused on this part that, in this network staff can easily manage their own sections and use the engineering department website for uploading their modules materials and their weekly assignments. There are lots of ideas in our mind about all the tasks to do and all the features to include in this model. All the features that are asked by the stakeholder must be included. So, they are always our top-level priorities. However, proper aims of our project are listed below:

- Problem domains:

A lot of problems might arise in every step of building this model. We would evaluate all the problem domains beforehand at the start of the project. The main thing before doing any further plan and implementation, it is very necessary to know what our project will behave, and what problem will arise during the work. So, before dive into the development, the little more time will be allocated to find the problem domain, and it will be done by taking interview with the Network technician of the computing department followed by the field visit. With the mindset of not having to face unnecessary difficulties, we would extract relevant problem domains during the elicitation phase.

Some problem domains, that we are focusing one are:

- Windows Server Configuration

- Authentication for staff and student

- Create/add/delete user account by administrator

- Network redundancy

- Backup data

- Network Performance

- Security

Note: Points will be explained under requirements specification

- Interview:

The brief does not contain enough information about network model and structure to be built. Taking an interview with Network technicians would be a great idea to add up to the requirement specification. This could also provide us the idea of some extensions to be added to the model.

Note: The interview question and the interview’s answer are under Elicitation Activities

- Evaluation of pre-installed network system:

Before boarding on the main project design and implementation part, we would estimate the network model of the computing department. This would lead us to the essential and desirable features of a network system. A similar model would help administrators, staff and students in a user-friendly way.

- Implementation of prototype Network:

In the implementation section, we install the router and configure it with a required protocol on the basis of 3-tier/layer cisco hierarchical model.

- Servers Verification and configuration:

There are different servers in the network uses for different purpose. For example, file server store files, data, and information, and provide data access to specific user. Web server store the data related to the website and it host the website of staff personal website (with module material) and   the computing department website. The authentication server check the privileges level for giving the special permission to accessing the web or file server.

- Access and privileges:

All the three types of user account viz. administrator, staff, and student, have different access and benefits. Administrators could create, add, and delete user accounts for staff, students, and administrators. They could also add and delete files from the file server. The tutor of a specific module can make change in any file they uploaded like editing, deleting, and uploading the studying material. And the student, get access to read their material which are contain in their X-drive.

- Network Administrator:

The administrator would have all the freedom to manage the network environment. Network administrators could remotely access all the servers.

- Testing:

After the entire configuration, the testing is done for making sure that the network is working

- Requirements Engineering

So, the following points describe the needs and requirements, which have to be done and, implemented accordingly, for the completion of the project.

- Elicitation Activities

- Interview Plan question

So, these are the question which was asked to the network technician to get the basic information about previous network, and further implementation requirements and many more.

| S.N | Interviewer (Ashok and Kaushal ghimire) | Interviewee (Mr. Narinder Singh Network Technician) | Questions |
| --- | --- | --- | --- |
| 1. |  |  | As you are the current network technician of the university, so would you please tell the complete information of existing system? |
| 2. |  |  | What is the reason of focusing network topology design using Cisco 3 layer hierarchical model? |
| 3. |  |  | Is there any kind of problem in security or scalability in previous network or any kind of low-sides, that you are currently facing on? |
| 4. |  |  | What about the communication and file sharing between other departments, do you want to communication between engineering and other departments? |
| 5. |  |  | As we need to install the network resources according to scalability, what would be the total number of end user or end devices in the network? For example, mobile phones, printers, laptop, Pc etc. |
| 6. |  |  | For sub-netting section which class of IP do you prefer? |
| 7. |  |  | Is there is any kind of restriction in network access for students? |
| 8. |  |  | For the backup do you want to backup entire data of the server or some particular portion? If some, then, what kind of data they are? |
| 9. |  |  | Which programming language can we use for this project? |
| 10. |  |  | What amount of bandwidth is providing for database server? |
| 11. |  |  | What is the capacity of hardware that runs the database server? |
| 12. |  |  | Which OS are you familiar with, is it ok, if we use windows server 2016 R2 for the DNS, active directory domain services, backup etc. |
| 13. |  |  | What fully qualified domain name do you want to make for the active directory and web server? For example, nothamptonuniversity.ac.uk, or any other |

- Interview finding

The answer of the questions that we have get after interviewing the network technician are listed below in tabular form with the respective S.N question of 9.1.1:

| S.N | Question of 9.1.1 (Reference S N) | Answer |
| --- | --- | --- |
| 1. | 1. | 1. Have only one single network needs to re-design properly. 2. Not, have proper IP Planning for network and host portion. 3. Less secure L2 devices. 4. Not have firewall’s redundant switches. |
| 2 | 2. | 1.Because of Security issue 2. Redundant Network 3. reliable 4.Scability 5. Easy for troubleshooting |
| 3. | 3. | Already in SN 1 |
| 4. | 4. | 1. Allow communication but, all the files sharing in the department will not be visible to other departments. |
| 5. | 5. | Total student lab pc- Around 800+20% Growth=960 Total staff-teacher- 40+20% Growth=48 Administration 20+20% Growth =24 40 Server farm 20+20% Growth =24 |
| 6. | 6. | Any implementable class of IP can choose for sub-netting. |
| 7. | 7. | 1. Students should only have access with in lab 2. If some time student needs to share the file then definitely then separate file-Server will be there, using that the file sharing can happen. 3. Network access from one department to another department to the students will not be allow |
| 8. | 8. | 1. Data which are very crucial one which is more related to the data generating and accessing or allocated for our web server. 2.students portals and whole like students management system 3.Files which are available in personal server and files which are shared In X drive For student. |
| 9. | 9. | Can use any programming language. |
| 10. | 10. | Total 50MBps Bandwidth |
| 11. | 11. | So for now let’s consider 16 core CPU and 64GB ram and 1TB hard disk drive will be There, let’s consider this as a specification for the required database server. |
| 12. | 12 | Definitely windows OS. And I don’t have any issue while using windows server 2000 R2 but I strongly recommend you to use latest version even if you are using Microsoft trial They allow 180 days for free. 2016 is ok for project as server. |
| 13 | 13 | For Active directory domain and web server the fully qualified domain should me uon.com and nothamptonuniversity.ac.uk respectively. |

- Other Problem domain

- Comparable Network System review

To design our network and for getting more ideas for designing the network topology more secure and reliable; we have checked out some poorly designed and highly secure currently running network topology in Google and try to improve our network by knowing the differences. There are some network topologies which we have compared and make note of its backside.

- Network Topology of  Salahaddin university campus

This is the network design for the Salahaddin university campus. So the network of this university is designed very poorly.

Fig 1: Salahaddin university network Topology

Description:

There is an only one layer-2 switch for the access layer in the network; the router routed the packet for the communication, and SVI is implemented there for inter Vlan routing protocols. Firewall and server are placed in an untidy manner. So, some positive sides and negative side of this network topology is tabled down below:

| S.N | Positive sides | Negative Aspect |
| --- | --- | --- |
|  | Less resources | Poorly design network |
|  | Low Costs | Less secure and instability |
|  | No wireless access | No- redundancy |
|  |  | Single point failure |
|  |  | Not-implemented Cisco 3 layer hierarchical model |
|  |  | No back-up devices |
|  |  | High chances of Domain collision |

So, by reading above table we can easily make decision on, why we should not follow this kind of topology for designing our network. The main reason for not selecting this network topology is the id does not fulfill our requirements and have negative side high than the positive sides. And in future it will arise lots of problem, and will be very difficult to troubleshoot.

- Secure campus network topology

Fig 2: Secure campus network topology

Description:

This is the fully secure, stable and redundant network topology. Here the devices are implemented by following Cisco 3 layer hierarchical model. And the positive and negative side of this network topology id tabled below:

| S.N | Positive sides | Negative Aspect |
| --- | --- | --- |
|  | Implemented Cisco 3 layer hierarchical model | High cost |
|  | Highly Secure, ACL, port security | High resources |
|  | Multiple router for increasing redundancy | More space needed and multiple wire |
|  | No single point of failure |  |

- Development Relevant Legislation

1. Data Protection Act:

The Data Protection Act (2018), UK states that everyone responsible for using personal data has to follow ‘data protection principles’ where the information should be transparent, accurate, up-to-date, and secure. Person who is about to give away his personal information should be aware of every field and instance where that information could be used. There is stronger legal protection for more sensitive contents related to race, ethnic background, political opinions, religious beliefs, trade union membership, genetics, and health and sex life.

2. Digital Economy Act:

The digital economy act encourages people to digitize every aspect from their sector. This act will Empower consumers and provide better connectivity so that everyone has access to broadband wherever they live to fit the digital future. This will also enable better public services using digital technologies and protect children from online pornography.

3. Computer Misuse Act:

In the digitally developing world, there is data saved in every digital sector. Those data could be stolen unethically by unauthorized personnel. The computer misuse act strictly prohibits any kind of harm, harassment, bully or robbery through digital medium. Online scam, robbery, and data stealing are acts that could lead to a big punishment. Any other relevant problem domain investigation data:

· Difficulty in keeping and maintaining records of each and every student because of the traditional based system.

· Quality of education could not be maintained due to an uneasy network system.

· Module activity tracks could not be kept on time for both tutors as well as students.

· Lack of communication medium between students and teachers.

- Requirements Specification

- Problem domain Description

A lot of problems might arise in every step of building this model. We would evaluate all the problem domains beforehand at the start of the project. The main thing before doing any further plan and implementation, it is very necessary to know what our project will behave, and what problem will arise during the work. So, before diving into the development, a little more time will be allocated to find the problem domain, and it will be done by taking an interview with the Network technician of the computing department followed by the field visit. With the mindset of not having to face unnecessary difficulties, we would extract relevant problem domains during the elicitation phase.

Some problem domains, that we are focusing one are:

- Windows server Configuration

- Network redundancy:

For the purpose that, if one path to reach the packets to the destination is blocked then, it will not be dropped, and find its other path and make the packets to reach the destination at any cost.

- Backup data :

When it comes to data and information, they are the most valuable assets of any organization or company. If any disaster happens to the data, then the company has to bear a heavy loss and may be shut down, if they have not stored their data in a safe place, and the process of saving the data, files, or any kind of e-documents is known as backup. So, any organization where information and data is the most important thing then the backup becomes the most important thing to save data if the original data is destroyed.

The backup can be set for one in a week, or on a monthly basis, or can be done daily. There is the limitation for the backup, its limits for 3-4 months. Basically, the backup of the data is stored in the remote area for the safety purpose and to make data available online if the lost data is needed immediately. In the backup process there are 2 kinds of backup processes, one is hot backup and other is known as cold backup.

- Security : On

- Functional Requirements

- Access Layer Requirements:

The access layer is the interface which enables end devices such as PCs, printers, modems and IP phones to connect to the rest of the system. At this layer, depending on the system requirement, network devices such as wired routers, switches, wireless access points and routers may be necessary. The access layer switch should support multiple speeds, VPN and rapid failover capability. Physical and logical Redundancy is introduced on the link between the active devices at this layer and the distribution layer. On each of these devices-switches and routers precisely, consideration may also be made for redundant power supply. Also, centralized management and wireless access point are required for wireless connectivity applications. One of the concepts that can be employed at this layer is the Virtual Local Area Network (VLAN) which enables logical grouping of end-stations that are physically dispersed on a network. Implementation of VLANs lead to ease of administration, confinement of broadcast domains, reduced broadcast traffic, and enforcement of security policies. 5.2.

- Distribution Layer Requirements:

The distribution layer is an intermediate layer between the core and the access layers. The increase in the use of bandwidth-intensive applications and services at the access leads to higher traffic volumes. Therefore, multiple access layer switch traffics are aggregated to the core layer. In view of this, high-performance devices with significant redundancy are required for system availability and stability. Moreover, parts of the required components are high-performance backplanes, high-density port modules, redundant power supplies and management modules. Also, the routing protocols to be employed in the network should be open standard. The network traffic can be efficiently managed at this layer with load balancing which can be used to define quality of service (QoS) required the VLANs.

- Core Layer Requirements :

The core layer also requires high-speed, high-performance devices with significant redundancy. This is required for the fast delivery of the aggregated traffic. Furthermore, core switches are designed with high-density modules of high-performance ports. There may be need for the core layer to support advanced multiprotocol label switching (MPLS) and multi-virtual routing and forwarding (Multi-VRF) protocols in order to enforce traffic separation and QoS policies. Also, gateway redundancy is essential to protect the network against a single point of failure and for traffic load balancing. Furthermore, for different vendors’ devices interoperability, implementation of HSRP is advised. Also, the routing protocols to be employed in the network should be open standard. The overall network architecture of a resilient campus network is depicted in Figure 6. Some detail are not presented for simplicity purpose. The 1st departmental access layer is emphasized and the concept can be applied to other departments depending on the requirement of each one of them. Moreover, EtherChannel employed to create logical link for the purpose of providing fault-tolerance and high-speed links between switches, routers and servers. Also, the configuration presented for the demilitarized zone (DMZ) can be extended to the server farm

- Performance Requirements

• High scale and reliability:

Mobility Master can centrally manage up to 100,000 clients, 10,000 APs and 1,000 mobility controllers. Mobility controllers become managed devices under the Mobility Master and handle the AP terminations.

• 24/7 always-on operations

Our network will work 24/7 without any disturbance in the performance. The extra routers and switches are installed and configured for backup as well the load-balancing purpose.

• Capacity

- Security Requirements

- Port Security:

It enables you to limit the number of MAC address allowed on a port, stopping MAC flooding attacks. MAC addresses can be learned by the switch or statically configured, and if there is a violation, you have the option of sending an alarm, disabling the port, or both. Be careful when deploying this feature when multiple MAC addresses are required behind a single port (for example, VMs, IP phones, unmanaged switches, etc.).

- DHCP snooping:

It stops IPv4 DHCP starvation attacks, in which an attacker repeatedly requests an address from a DHCP server until no more addresses are available, causing a denial of service to other users. It also prevents rogue DHCP servers by only allowing replies from a trusted server on a trusted switch port, typically the uplink ports to the aggregation layer.

- ARP protect:

It stops man-in-the-middle attacks caused by ARP cache poisoning, by verifying the source IP-MAC binding information in the DHCP snooping table. This prevents hosts from sending spoofed ARP messages to fool devices into sending traffic to the wrong address

- BPDU Protection:

It Prevents loops in the network by putting a non-trunk port into a disabled state for a specified amount of time when it receives a BPDU from another device. This is normally caused by a rogue device being connected to an access port on a switch.

- Testing Requirements

- Security Testing:

After the completion of the project, the network will be tested from different view aspects, so the terms that are going to be tested for are:

- Port security testing:

This test is done to make sure that a particular user’s device is connected to port. For example, if the device of SALE is trying connecting its device in the switch port of manager, then it should not give access to the sales pc.

- Performance Testing:

For the better speed and reliability, this test has to be done in the network. This is the process in which, a network is tested on the basis of speed quality, stability to measure the performance as well as the QoS of a Network.

- Hardware and Software Requirements

The following devices and the software are needed to complete the project; the table shows the device name, device model type: i.e. Hardware or software, Purpose of using, and cost.

| S.N | Equipment | Model/ version | Type | Purpose | Quantity | Total Cost |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cisco Router | C2691 | Hardware |  | 2 | $9202 |
|  | Cisco L2 Switch | C2960 | Hardware | For access layer, to manage layer 2 switching protocols. | 3(At this time) | $10254 |
|  | Cisco L3 Switch | 3560 | Hardware | For the inter VLAN routing and manage the traffic to uplinks routers for communication | 2 | $5505 |
|  | PC |  | Hardware | For user to do their task | 100-200 min | $100000 |
|  | Hub |  | Hardware |  | 1 | $200 |
|  | Wire | Cat6 |  | For the connection between devices |  | $2000 |
|  | Windows server | 2016 | Software | For hosting website, providing DHCP to the end devices in the network, manages Active directory, Manage DNS, etc. | 1 | Free |
|  | Windows OS | 10 | Software | Operating system needs for user to do their job. | As the 4 | Free |

- Network and Web design

To enhance the overall user experience and organizational Network systems, the Network that we have designed for this project and the web pages that we are going to hosting in the web server are broadly described below:

- Network Topology design

The figure 3 is the picture of network topology that we have designed for this project.

The main point has been focus on this topology is listed below:

- Follow Cisco 3-layer hierarchical model (See here what is Cisco 3-layer hierarchical model:  )

- Well designed

- Make note of scability

- VPN connection between site1 and site 2

- Maintained redundancy

- Provides high performance

- Load balancing implementation

Figure-3: Network Topology for engineering departments

Description:

There are two sites in this network, one is network site 1 for internal network which will placed in the collage building, and, another one is network site 2, which is located in remote area. And the Virtual private network (VPN) technology is implemented in between internal network and remote network for the communication. To be more specific, the remote connection is for Network Administrator for accessing the Servers from remote area, as per the requirements.

The one wire is connected from the ISP cloud to the hub1 and then the Core routers get the connection from ISP through the Hub1. Then, the core layer router is then connected to the distribution layer layer-3 switch. The distribution layer balance the traffic came from Assess layer and core layer as well as is do inter vlan routing for the communication between staff , student and administration. Since, some restrictions will be implemented for students and staff; they do not have full access to the server sites. The windows server 2016 is connected to the distribution layer-3 switch1, which will manage, DNS (For resolving the IP address), DHCP (to provide dynamic IP address to the end devices) , active directory , web services, FTP (file transfer protocols ) etc. Wireless Access point is connected to the distribution layer-3 switch2 for providing wireless access to the end devices and do authentication using Radius server. And at last the access layer switch is placed for users.

- Website application designed mockup

Student

STAFF

Admin

- Implementation

- Network Implementation

- VLAN Implementation:

Vlan stands for virtual local area network. In different LAN segments vlan divides the group of user on different network.

In our network topology we need to configure Vlan on distribution layer-3 and access layer-2 switches. So we configure the vlan on those switches as shown in below imaged:

Command for creating vlan in Config mode and screenshot of vlan database and port assigned to it:

L2-Sw3(config)#vlan 10

L2-Sw3(config-vlan)#name student

L2-Sw3(config-vlan)#exit

L2-Sw3(config)#vlan 20

L2-Sw3(config-vlan)#name staff

L2-Sw3(config-vlan)#exit

L2-Sw3(config)#vlan 30

L2-Sw3(config-vlan)#name Administration

L2-Sw3(config-vlan)#exit

L2-Sw3(config)#vlan 40

L2-Sw3(config-vlan)#name server

L2-Sw3(config-vlan)#exit

L2-Sw3(config)#vlan 99

L2-Sw3(config-vlan)#name management

L2-Sw3(config-vlan)#

This creates the vlan in the vlan database in switches, do same on all access layer and distribution layer switches:

- VTP configuration:

VTP stands for VLAN trunk protocol. And it helps to reduce time of administrator by propagating VLAN database information to its clients switches if the configuration is made on vtp server switch.

So for security reason all the switches of access layer and distribution layer, the VTP mode Transparent is implemented with the domain name uon.com, instead of mode server and client. If another switch is connected in the network in future with the higher revision number then that switch become the VTP sever and all the vlan database will be synchronize on the bases of that new switch and problem may arise at that time, so VTP mode server and client is strictly avoided in this project.

Command for configuring VTP on configuration mode:

L2-Sw1(config)#vtp mode transparent

L2-Sw1(config)#vtp domain uon.com

L2-Sw1(config)#end

L2-Sw1#

- Trunking Protocol implementation:

The main reason of implementing trunking protocols is, so that all the Vlan traffic can move through on switch port. For example, by default the switch port is in mode access which can pass the traffic of only one vlan, so, to avoid this issue we implement the trunking protocols on the Ethernet port, so that all the vlan’s traffic can go through the switch port.

Command for configuring VTP on configuration mode:

L3Sw1-Dist(config)#inter range eth1/0-3

L3Sw1-Dist(config-if-range)#switchport mode trunk

L3Sw1-Dist(config-if-range)#switch trunk native vlan 99

L3Sw1-Dist(config-if-range)#end

- Vlan Traffic load balancing (VLAN HSRP):

HSRP stands for Hot Standby Routing protocols. Basically, HSRP is done for redundant purpose. But in this project, HSRP implemented in such a way that, it balance the load of distribution layer switch. For example, for vlan 10 and 20 the traffic will move through the L3-switch1 whereas the traffic of vlan 30 and 40 will travel through L3-swich2. If one switch goes down then other switch works as a main switch for that vlan.

Command for configuring HSRP on configuration mode in vlan interface:

L3Sw1-Dist(config)#inter vlan 10

L3Sw1-Dist(config-if)#no shut

L3Sw1-Dist(config-if)#ip add 172.17.0.3 255.255.252.0

L3Sw1-Dist(config-if)#standby 1 ip 172.17.0.1

L3Sw1-Dist(config-if)#standby 1 priority 121

L3Sw1-Dist(config-if)#standby 1 preempt

L3Sw1-Dist(config-if)#inter vlan 20

L3Sw1-Dist(config-if)#no shut

L3Sw1-Dist(config-if)#ip add 172.17.4.3 255.255.255.192

L3Sw1-Dist(config-if)#standby 1 ip 172.17.4.1

L3Sw1-Dist(config-if)#standby 1 priority 121

L3Sw1-Dist(config-if)#standby 1 preempt

L3Sw1-Dist(config-if)#inter vlan 30

L3Sw1-Dist(config-if)#no shut

L3Sw1-Dist(config-if)#ip add 172.17.4.67 255.255.255.224

L3Sw1-Dist(config-if)#standby 1 ip 172.17.4.65

L3Sw1-Dist(config-if)#standby 1 priority 151

L3Sw1-Dist(config-if)#standby 1 preempt

L3Sw1-Dist(config-if)#inter vlan 40

L3Sw1-Dist(config-if)#no shut

L3Sw1-Dist(config-if)#ip add 172.17.4.99 255.255.255.224

L3Sw1-Dist(config-if)#standby 1 ip 172.17.4.97

L3Sw1-Dist(config-if)#standby 1 priority 151

L3Sw1-Dist(config-if)#standby 1 preempt

- DHCP Helper IP VLAN interface:

In this network, all the vlan’s end users will get an IP address from DHCP server which is configured on the windows server 2016. So, for accessing that scope pool we need to say that, when each vlan request for the DHCP, the gateway device must redirect it to the DHCP server, so that it get its IP address. So, for that, on each vlan interface we need to give ip-helper IP address of DHCP server so in this cast the DHCP server’s IP address is 172.17.4.100.

Command for DHCP ip helper address on configuration mode in vlan interface:

L3Sw1-Dist(config)#inter vlan 10

L3Sw1-Dist(config-if)#ip helper-address 172.17.4.100

L3Sw1-Dist(config-if)#no shut

L3Sw1-Dist(config-if)#inter vlan 20

L3Sw1-Dist(config-if)#ip helper-address 172.17.4.100

L3Sw1-Dist(config-if)#no shut

L3Sw1-Dist(config-if)#inter vlan 30

L3Sw1-Dist(config-if)#ip helper-address 172.17.4.100

L3Sw1-Dist(config-if)#no shut

- EtherChannel Port channel configuration

Basically, the Etherchannel is configuring for binding the multiple physical ports into a single logical ports. Specially, this is done to increase the bandwidth capacity in a network, by combining all the ports and, if one physical link goes down then other link works to travel the traffic. So, in this case we have configured, 3 Ethernet port into a single logical port to increase bandwidth and provide redundancy.

Command for EtherChannel on configuration mode:

L3Sw1-Dist(config)#inter range eth4/1-3

L3Sw1-Dist(config-if-range)#channel-protocol lacp

L3Sw1-Dist(config-if-range)#channel-group 10 mode active

L3Sw1-Dist(config-if-range)#exit

L3Sw1-Dist(config)#inter port-channel 10

L3Sw1-Dist(config-if)#no shut

L3Sw1-Dist(config-if)# switchport trunk native vlan 99

L3Sw1-Dist(config-if)# switchport trunk allowed vlan 1-1001

L3Sw1-Dist(config-if)# switchport trunk encapsulation dot1q

L3Sw1-Dist(config-if)# switchport mode trunk

- Dynamic routing configuration (using EIGRP):

- NAT implementation

- VPN implementation

- Windows server Implementation

In this project the windows server 2016 has been urged by our network technician. So, the function such as ADDS, DNS, FILE Server, Web Server, DHCP, creating shared folder, creating organizational unit, groups, users, and managing these user, implementing group policies, will be configured on this web server. So, first install Windows server 2016:

The process to install the windows server is:

- Firstly, select language, English is selected in this case.

- Then, click to Big Install button

- After that, Select OS you want to install and click on Next

- Check the  license term form and click next, then choose custom installation

- After that select disk and install the Server

The following images drive you toward the installation:

Static ip providing:

After installing windows server 2016 successfully, the static IP address must provide to the Network Adaptor, in this case, the ip of 172.17.4.100 ip address has been given to the windows server and the same ip address is use as DNS server ip address for resolving ip address.

To do that, follow the steps:

- Press Win+R Key and type ncpa.cpl to open control to view network adaptor

- Then, select the Ethernet adaptor -> (TCP/IPv4) ->Properties

- Then, the below tab will open, and provide ip static as following ,

Fig 4: Static ip configuration

Server computer Name Change:

For the easily identification of this server, the server computer name has to be changed. In this case the domain name UONSERVER has been chosen. So to change server name follow the following steps:

- Click windows => Server manager => Local server => Computer name

- System properties will pop up and the, click change to change computer name, and in this case the server/computer name is UONSERVER.

- ADDS (Active Directory Domain Services)

ADDS is an Active Directory Domain Service, which is foundation for distributed networks built on Win Server (2000), Win Server (2003) and Microsoft windows Server 2008 operating system (OS) that use domain controllers. Or we can also define this as Active Directory Domain Service (ADDS) is a specific service in an Active Directory that allow administrator/admin to organize/manage and store information about resources from a network or from an application data in a distribute database server. ADDS provides some of importance service and they:

- Domain services

- Certification services

- Lightweight services

- Directory federation Services

- Right Management

For the installation of AD DS go to server manager => Manage => Add roles and features, and then follow the following steps:

- A new window will show Heading of “Before You Begin”; simply click Next button.

- After that, it goes to another Link  on same window with heading “Select Installation Type” and selection the option “Role-based or feature-based installation’’: And click on Next Button

- On Link with heading “Select destination server”, choose “Select server form server pool” And then select server. Then, click Next

- On this windows Check or select “Active directory Domain services”, a new windows will popup showing that ask us to install Additional features: click on Add features And again click on Next button Again Select Next Button, and again Next,  Then click Install

- The ADDS is Installed Now but not become Domain controller.

Creating Domain Controller

Step 1: So, after, installing Active directory by following above steps, we need to make this server a domain controller. For that, click on Yellow Triangle icon near manage and click on “Promote this server to a domain controller”

Fig 5: 4.2.1 step 1

Step 2: After clicking the above said link, the Deployment Configuration Window will show up:

Fig 6: 4.2.1 step 2

So, here, there are three (3) options on deployment: Configure window, first two are related Existing forest, last one creates a new Root domain.

Existing Forest: Basically, this operation mainly uses for backup purposes, and administrator’s login details will be required to the user.

Add a new Forest: When the Domain controller is configured for the first time in the windows server environment, the root domain name must be defined to connect other client’s computer into the domain, which will be typed in this field. In this case, the root domain name is uon.co

Step 3: After step 2, the functional level must be defined for the newly created forest and root domain. So here we choose, Windows server 2016 on both functional levels as shown in figure. Since the DNS and GC (Global catalog) is mandatory for this installation, these options cannot be uncheck.  At last, we need a password to configure for restoration if anything goes wrong in future as shown in figure and click Next.

Fig 7: 4.2.1 step 3

Step 4: On this step, just click next to go another page because, by default the DNS is needed to resolve the root domain of the server, so cannot create a delegated DNS, and the option is also blocked for check, so simply click next.

Fig 8: 4.2.1 step 4

Step 5: In this step the Name of NetBIOS has to be verify here, by default the name of the NetBIOS is the first name of the Root name before .com. so, in this case, it will be UON. It can be changed if, it is necessary.

Step 6: In this step we need to provide the location for AD DS database, SYSVOL and Log files, by default, the locations are declare by itself there. So, the database and log files will be stored in the folder in C: Drive in the location C:\Windows\NTDS, and the SYSVOL folder in location C:\Windows\SYSVOL.

Fig 9: 4.2.1 step 6

Step 7: After completing till step 6, Step 7 is the last step for the installation of Active directory and make it a domain controller. So, this step is just to check prerequisites, and if step 1 to 6 has been configured correctly, then click on Install Button to create a domain controller. After the completion of Step 7, the pc will automatically reboot.

After step 1 to 7 complication, the installation AD DS is finished successfully without an error. So, to open Active directory, press WIN+R and type dsa.msc command, and window will be opened like below,

So, 2nd is the active directory windows here you can create organizational unit, groups, users, and managing these user, implementing group policies etc.